- MobileL3 (v13.4.0) Protocol Modules for TTCN-3 Toolset with TITAN, Function Description

Abstract

This is the Description for the MobileL3 v13.4.0 protocol module. The MobileL3v 13.4.0 protocol module is developed for the TTCN-3 Toolset with TITAN. This document should be read together with Product Revision Information [5].

Contents

1	Functionality	2

1.1	Implemented protocols	2

1.2	Modifications/deviations related to the protocol specification	2

1.2.1	Unimplemented and Implemented Messages, Information Elements and Constants	2

1.2.2	Ericsson-specific changes	3

1.3	Backward incompatibilities	3

1.4	System Requirements	3

2	Usage	4

2.1	Installation	4

2.2	Configuration	4

2.3	Examples	4

3	Interface description	4

3.1	Top Level PDU	4

3.2	Encoding/decoding and other related functions	4

3.2.1	Implemented encoding and decoding functions	4

4	Terminology	6

4.1	Abbreviations	6

4.2	Terminology	7

5	References	7

6	Change Information	8

6.1	R1A	8

# Functionality

The MobileL3 v13.4.0 protocol module implements the message structures of the related protocol [6] in a formalized way, using the standard specification language TTCN-3. This allows defining of test data (templates) in the TTCN-3 language and correctly encoding/decoding messages when executing test suites using the Titan TTCN-3 test environment.

The MobileL3 v13.4.0 protocol module uses Titan’s RAW encoding attributes [1] and hence is usable with the Titan test toolset only.

## Implemented protocols

This set of protocol modules implements a subset of protocol messages and constants of the Mobile L3 protocol. It includes GMM, SM, RRM, SMS and SS. SM and GMM are based on 24.008 v13.4.0 (see [4]), RRM are based on 44.018 v13.0.0 (see [6]), SMS are based on 24.011 v13.0.0 (see [7]) and 23.040 v13.0.0 (see [8]), and SS are based on 24.080 v13.0.0 (see [9]) with the modifications specified in 3.2.

## Modifications/deviations related to the protocol specification

### Unimplemented and Implemented Messages, Information Elements and Constants

#### Messages for mobility management (MM)

All the messages are implemented according to Table 9.2.1 and 10.2 of 24.008 (see [4]).

#### Messages for circuit switched call control (CC)

All the messages are implemented according to Table 9.54 and 9.3 of 24.008 (see [4]).

#### GPRS Mobility Management (GMM) Messages

All the messages are implemented according to Table 10.4 of 24.008 (see [4]).

#### GPRS Session Management (SM) Messages

All the messages are implemented according to Table 10.4a of 24.008 (see [4]).

#### Common Information Elements (CommonIE)

All the information elements implemented according to 10.5.1 of 24.008 (see [4]).

#### Radio Resource Management (RRM) messages

Some of the messages that are used are implemented according to table 9.1.1 of 44.018 (see [6]).

#### Short Message Service (SMS) messages

All the CP-messages are implemented according to 7.2 of 24.011 (see [7]).

All the RP-messages are implemented according to 7.3 of 24.011 (see [7]).

All the TPDU-messages are implemented according to 9.2.2 of 23.040 (see [8]).

#### Supplementary Service Management (SS) messages

All the SS-messages are implemented according to table 2.1 of 24.080 (see [9]).

### Ericsson-specific changes

None

## Backward incompatibilities

None

## System Requirements

Protocol modules are a set of TTCN-3 source code files that can be used as part of TTCN-3 test suites only. Hence, protocol modules alone do not put specific requirements on the system used. However, in order to compile and execute a TTCN-3 test suite using the set of protocol modules the following system requirements must be satisfied:

- Titan TTCN-3 Test Executor version CRL 113 200/5 R4A (5.4.pl0) or higher installed. For Installation Guide see [2]. Please note: This version of the test port is not compatible with Titan releases earlier than CRL 113 200/5 R4A.

# Usage

## Installation

The set of protocol modules can be used in developing TTCN-3 test suites using any text editor; however, to make the work more efficient a TTCN-3-enabled text editor is recommended (for example nedit, xemacs). Since the MobileL3 protocol is used as a part of a TTCN-3 test suite, this requires TTCN-3 Test Executor be installed before the module can be compiled and executed together with other parts of the test suite. For more details on the installation of TTCN-3 Test Executor see the relevant section of [3].

## Configuration

None.

## Examples

None.

# Interface description

## Top Level PDU

The top level PDUs are the TTCN-3 records PDU_L3_MS_SGSN, PDU_L3_SGSN_MS, PDU_ML3_NW_MS, PDU_ML3_MS_NW.

## Encoding/decoding and other related functions

This product also contains encoding/decoding functions, which assure correct RAW encoding of messages when sent from TITAN and correct RAW decoding of messages when received by TITAN.

### Implemented encoding and decoding functions

Name	Type of formal parameters	Type of return value

enc_PDU_L3_MS_SGSN	PDU_L3_MS_SGSN	octetstring

enc_PDU_L3_MS_SGSN_fast	in PDU_L3_MS_SGSN,
	out octetstring

dec_PDU_L3_MS_SGSN	octetstring	PDU_L3_MS_SGSN

dec_PDU_L3_MS_SGSN_backtrack	in octetstring, 	integer (0: success,
	out PDU_L3_MS_SGSN	1: decoding failed)

enc_PDU_L3_SGSN_MS	PDU_L3_SGSN_MS	octetstring

enc_PDU_L3_SGSN_MS_fast	in PDU_L3_SGSN_MS,
	out octetstring

dec_PDU_L3_SGSN_MS	octetstring	PDU_L3_SGSN_MS

dec_PDU_L3_SGSN_MS_backtrack	in octetstring,	integer (0: success,
	out PDU_L3_SGSN_MS	1: decoding failed)

enc_PDU_ML3_NW_MS	PDU_ML3_NW_MS	octetstring

enc_PDU_ML3_NW_MS_fast	in PDU_ML3_NW_MS,
	out octetstring

dec_PDU_ML3_NW_MS	octetstring	PDU_ML3_NW_MS

dec_PDU_ML3_NW_MS_backtrack	in octetstring, 	integer (0: success,
	out PDU_ML3_NW_MS	1: decoding failed)

enc_PDU_ML3_MS_NW	PDU_ML3_MS_NW	octetstring

enc_PDU_ML3_MS_NW_fast	in PDU_ML3_MS_NW,
	out octetstring

dec_PDU_ML3_MS_NW	octetstring	PDU_ML3_MS_NW

dec_PDU_ML3_MS_NW_backtrack	in octetstring, 	integer (0: success,
	out PDU_ML3_MS_NW	1: decoding failed)

enc_SS_FacilityInformation	SS_FacilityInformation	octetstring

dec_SS_FacilityInformation	octetstring	SS_FacilityInformation

dec_SS_FacilityInformation	in octetstring, 	integer (0: success,
	out SS_FacilityInformation	1: decoding failed)

enc_TPDU_RP_DATA_MS_SGSN_fast	in TPDU_RP_DATA_MS_SGSN
	out octetstring

dec_TPDU_RP_DATA_MS_SGSN_backtrack
	in octetstring 	integer (0: success,
	out TPDU_RP_DATA_MS_SGSN	1: decoding failed)

enc_TPDU_RP_DATA_SGSN_MS_fast	in TPDU_RP_DATA_SGSN_MS
	out octetstring

dec_TPDU_RP_DATA_SGSN_MS_backtrack
	in octetstring 	integer (0: success,
	out TPDU_RP_DATA_SGSN_MS	1: decoding failed)

enc_TPDU_RP_ACK_MS_SGSN_fast	in TPDU_RP_ACK_MS_SGSN
	out octetstring

dec_TPDU_RP_ACK_MS_SGSN_backtrack
	in octetstring 	integer (0: success,
	out TPDU_RP_ACK_MS_SGSN	1: decoding failed)

enc_TPDU_RP_ACK_SGSN_MS_fast	in TPDU_RP_ACK_SGSN_MS
	out octetstring

dec_TPDU_RP_ACK_SGSN_MS_backtrack
	in octetstring 	integer (0: success,
	out TPDU_RP_ACK_SGSN_MS	1: decoding failed)

enc_TPDU_RP_ERROR_MS_SGSN_fast	in TPDU_RP_ERROR_MS_SGSN
	out octetstring

dec_TPDU_RP_ERROR_MS_SGSN_backtrack
	in octetstring 	integer (0: success,
	out TPDU_RP_ERROR_MS_SGSN	1: decoding failed)

enc_TPDU_RP_ERROR_SGSN_MS_fast
	in TPDU_RP_ERROR_SGSN_MS
	out octetstring

dec_TPDU_RP_ERROR_SGSN_MS_backtrack
	in octetstring 	integer (0: success,
	out TPDU_RP_ERROR_SGSN_MS	1: decoding failed)

enc_RPDU_SGSN_MS_fast	in RPDU_SGSN_MS
	out octetstring

dec_RPDU_SGSN_MS_backtrack	in octetstring 	integer (0: success,
	out RPDU_SGSN_MS	1: decoding failed)

enc_RPDU_MS_SGSN_fast	in RPDU_MS_SGSN
	out octetstring

dec_RPDU_MS_SGSN_backtrack	in octetstring 	integer (0: success,
	out RPDU_MS_SGSN	1: decoding failed)

# Terminology

## Abbreviations

3GPP	3rd Generation Partnership Project

GMM	GPRS Mobility Management

GPRS	General Packet Radio Service

IE	Information Element

L3	Layer 3

PDU	Protocol Data Unit

SM	Session Management

TTCN-3 	Testing and Test Control Notation version 3

MM	Mobility Management

CC	Circuit Switched Call Control

RRM	Radio Resource Management

SMS	Short Message Service

SS	Supplementary Service Management

## Terminology

TITAN	TTCN-3 Test Executor (see [3]).

# References

- ETSI ES 201 873-1 v4.5.1 (2013-04) 
The Testing and Test Control Notation version 3. Part 1: Core Language

- 1/ 198 17-CRL 113 200/5 Uen 
User Guide for TITAN TTCN-3 Test Executor

- 2/198 17-CRL 113 200/5 Uen
Programmer’s Technical Reference for Titan TTCN–3 Test Executor

- 3GPP TS 24.008 V13.4.0 (2015-12),
 3rd Generation Partnership Project; Technical Specification Group Core Network and Terminals; Mobile radio interface Layer 3 specification; Core network protocols; Stage 3 (Release 13)

- 109 21-CNL 113 832-1 Uen
MobileL3 (v13.4.0) Protocol Modules for TTCN-3 Toolset with TITAN, Product Revision Information

- 3GPP TS 44.018 V13.0.0 (2015-12), 
3rd Generation Partnership Project; Technical Specification Group GSM/EDGE Radio Access Network; Mobile radio interface Layer 3 specification; Radio Resource Control (RRC) protocol; (Release 13)

- 3GPP TS 24.011 V13.0.0 (2015-12), 
3rd Generation Partnership Project; Technical Specification Group Core Network and Terminals; Point-to-Point (PP) Short Message Service (SMS) support on mobile radio interface (Release 13)

- 3GPP TS 23.040 V13.0.0 (2015-12),  
3rd Generation Partnership Project; Technical Specification Group Core Network and Terminals; Technical Realization of the Short Message Service (SMS) (Release 13)

- 3GPP TS 24.080 V13.0.0 (2015-12), 
3rd Generation Partnership Project; Technical Specification Group Core Network and Terminals; Mobile radio interface layer 3 supplementary services platform; Formats and coding (Release 13)

# Change Information

## R1A

Initial implementation